# NT Gmodem Service

Version 1.0

10/02/97

# Gmodem Service

## Overview

Gmodem service is designed to provide a secure login and data transport for remote users to an enterprise NT Server.  The protocol is designed for operation over wireless systems and aggressively moves the data from the remote unit to the server.

The secure login process begins when the pen unit transmits a properly formatted, properly encrypted packet to the server.  The server then decrypts the packet and verifies that the encrypted packet contains a valid user and password.  If and only if the user and password is valid is a challenge issued to the remote unit.  The remote unit must decrypt the challenge packet, modify it, encrypt it again and retransmit the exact response packet to the server to complete the login process.  Since each login attempt is unique in time, this prevents "playback" attacks against the system.

The service itself is designed using a client server model.  In the MobilePoint case, the remote unit (pen) is the client and the NT Server is the server in the model.  The client generates requests for the server and the server fills the requests.  Currently, only a few server functions are implemented.  These functions are "Upload Market Intelligence", "Upload Section", "Download Section" and "Hang up".

The gmodem protocol employs data, command and file transfer type packets.  Using a data channel allows the client-server communication to occur quickly and with very little overhead, especially when compared to using a file transfer protocol for client-server communication.  The protocol is adaptive and will aggressively reduce packet sizes when transmitting over a noisy channel.

The sections uploaded to the server are stored as GZIP (*.gz) format files.  Entries are made in the section file table when a section is either uploaded or overwritten.  Sections can be removed from the file table simply by deleting the associated *.gz file.

## Quick Install

WARNING:  administrator privileges are required to install this service.  This service must be able to validate login requests.

Insert disk into floppy drive A:

Set the current drive to the desired installation drive.  For example, if "D:" is the desired installation drive, type D:<CR>.

Install the software using the command  A:\install.bat<CR>.  Installation will leave you in the default installation directory of  \Program Files\gmodemService.

Type "gmodemService -install Com[1 - N] <CR>", where Com1 means install on COM1, Com2 means install on COM2, Com3 means install on COM3, etc.

The services will be installed with the modem configurations found in gmodemService.ini (US Robotics) and with a directory structure to match the queue directory structure in the file section.cfg.

The service should be up and running at this point.

Create valid usernames and passwords to match the remote users.  See "User Accounts" for more information.

## Modems

#### U.S. Robotics Sportster 28.8/33.6

The service was developed using the U.S. Robotics Sportster modem.  Thus, the default installation configuration is configured for the U.S. Robotics Sportster 28.8/33.6 modem.

#### Settings (Default settings for US Robotics Sportster)

The communication settings for the service and the modem are in a file named gmodemService.ini, normally found in the installation directory.  The default installation directory is "C:\Program Files\GmodemService".  This file contains the settings controlling the baud rate, timeout values, maximum data block size, flow control, security level, and specific setup commands for the US Robotics Sportster.  Setting up other modems with a similar command set in the same order should create a viable initialization command set for the new modem.

For a complete description of the commands and actions, see the US Robotics Sportster manual.  As a general guideline for setting up other modem types, here is essentially the process:

Reset modem to known factory default values.

Turn Echo OFF.

Set the DSR line to On.

Force modem carrrier line to FOLLOW the carrier.

Set the modem sound on, but keep it quiet.

Force the baud rate between modem and computer to remain (independent of actual carrier baud rate).

Disable MNP-5 compression

Enable V.42bis compression

Disable software flow control (Xon/Xoff).

Enable hardware flow control (CTS/RTS).

Save the profile in NVRAM (non-volatile memory).

Make the saved profile the default power-up profile.

#### DIP switch

US Robotics Sportster modems are shipped with DIP switch 5 in the down position.  The down position specifies that the auto-answer function is to be turned OFF on modem initialization.  This switch will take precedence over the settings in the NVRAM.  For the auto-answer function to be enabled, this switch must be in the up position.  Failure to move this switch to the "up" position will cause the auto-answer function to be disabled with the slightest power blink.  Some types of line errors will also cause the modem to "reset" itself and disable auto-answer.  This switch must be up!

#### Modem Configuration Files

The primary modem configuration file is called gmodemService.ini and is stored in the service installation directory, usually "C:\Program Files\gmodemService\gmodemService.ini".  This file controls the setup of the modem during the installation.  The installation file is not used after the initial installation of the service.  After installation, the settings and command strings for the modems are read from the registry.

Modifying the modem initialization commands can be accomplished in 2 ways.  The "safe" way is to de-install the service for the com port in question, edit the gmodemService.ini file and re-install the service for the desired com port.  The second way involves using the registry editor.  The modem initialization commands are located in the following key:

HKEY_LOCAL_MACHINE/SOFTWARE/MobilePoint/

GmodemService/ComN/Commands.

In order to change the settings using this method the gmodem Service for the com port to be modified must first be shut down using the Service Manager for the NT Server.  After shutting down the manager user the NT Registry editor to change the values of the initialization command strings.  After the changes have been made, re-start the gmodem Service for the COM port just modified.

#### Using other modems

The primary considerations are auto answer, command echo, baud rate, flow control, timeout values, compression settings, and power-up/reset behavior.  These commands are discussed in the settings section also. Re-iterating:

Auto-answer should be ON.  ATS0=1 will set this on most modems.

Echo must be OFF.  Failure to set this to OFF will create modem command timeouts.  Normally this is ATE0.

Time out value should be 2000-3000 milliseconds.  May want to increase the value when many "time-out" failures are occurring.  Smaller values mean faster throughput only when time-outs are not occurring.  If the time-out value is set too small, it will induce packet time-out when time-outs should not be happening.  I would recommend leaving this alone.

The baud rate is currently set at a default value of 57600.  This is about as fast as the remote unit can reliably read data from the serial port on remote units which are used to connect to Sun or Unix systems (38400 baud rate limit).  This number can probably be increased to 115200 on both the remote units and the NT Server, but ONLY IF connections are restricted to the NT Server.

Flow control should be set to hardware on the NT Server and the modem.  This means using the CTS/RTS control lines.  Software control MUST be disabled on both the NT Server AND on the modem, EVEN IF NO FLOW CONTROL IS USED.  Gmodem requires an 8-bit channel, meaning all 255 character values must be valid.

Use V.42bis compression when ever possible.  This compression scheme is better than MNP-5 and will not attempt to re-compress compressed data streams.  MNP-5 will compress compressed data streams, making the stream longer.  Disable MNP-5 compression since most of the files transmitted are already compressed.

Save the settings in the non-volatile memory and make the settings the default power-up settings.  The reason is that even though the NT Server may be powered through UPS, the modems may not be.  Any power glitch can cause the modem to reset.  Thus, the power up profile should match the desired profile.

# Directory Configurations

## Directory Configuration Files

#### Configuration File

The file "section.cfg" is used to control the section directory configuration.  This file has the settings for the section upload directory, the material request directory, receive log directory (Market Intelligence), the fax document directory, user root directory (not used) and finally an error directory.  These are the directories associated with the HQ Link application on the MobilePoint remote pen unit.

#### Default queue structure

#### The queue structure is controlled by the configuration file gmodemService.cfg.  This configuration file sets up incoming and outgoing directories, magic phrases, client command timeout values, the greeting phrase, commands to execute upon receipt of a file or immediately after downloading a file and more.  Many of these settings are used on the Unix version of the server to control order and update processing.  The NT server does not actually do order processing or update processing, but the hooks are installed to support those types of functions.  The directories in this file are primarily associated with the OrderGuide application on the MobilePoint remote pen unit.

#### Modification of the queue structure is a simple matter of using your favorite editor or wordpad to edit the file gmodemService.cfg and changing the desired directory name.  As an example, to move the default incoming queue, simply replace INQUEUE=\queue\incoming with the new queue name, such as INQUEUE=\myFavoriteQueue\NotTheDefault.

## User Accounts

#### Magic Phrase

The magic number or magic phrase is used by the secure login function to decrypt the incoming login request.  This facilitates the segregation of users on the same NT Server.  As an example, suppose Corporation X is assigned the magic phrase "X is the best" and Corporation Y is assigned the magic phrase "Why ask Why?".  If  a user from corporation Y accidently (or on purpose) attempted to login on Corporation X's communication port the encrypted login packets would be invalid on Corporation X's communication port.  The failure would be logged, the user from Corporation Y would not get a response.

The magic phrase controls whether or not the user even gets to attempt a login.  If and only if the magic phrase is valid will the login request packet be checked for validity after decryption.  If the magic phrase is correct, the user ID and password will then verified for the NT server in question.  The user ID and password MUST BE VALID for a response to be issued to the remote pen unit.

If a remote pen user has either an invalid or corrupted magic phrase, the login attempt WILL ALWAYS FAIL.  The user will have carrier, but the login attempt will always timeout.

If the remote pen user has a VALID magic phrase, but the user ID has either not been created or the password is invalid, the login attempt WILL ALWAYS FAIL.  Again, the user will have carrier, but the login attempt will always timeout.

The user id, group, password and magic phrase in the following example are simply examples and do NOT represent the actual settings.  The actual settings will be determined for each individual installation of the service.

#### Changing the Magic Number

The magic number can be changed either by modifying the registry values for the magic number or by editing the gmodemService.cfg file prior to installation.  This file contains a setting called MAGIC_NUMBER.  Edit this line to set the private magic number for this service.  The users of this service MUST have IDENTICAL magic phrases.  The magic phrase for the remote units is set by setting the MobilePoint environment variable NPCommMagic.

On Server:	MAGIC_NUMBER=E-Case Works!!! in the gmodemService.cfg file, PRIOR to installation.

On Pen:	NpCommMagic= E-Case Works!!! in all 3 environ.ini files.

The important settings in the environ.ini file on the MobilePoint remote pen unit used by the HQ Link application are:

# Set the User Name (User ID)

NPCommUserId=ABC

# Set the User Group (Domain or NT Server Name)

NPCommGroupId=MobilePoint

# Set the user password

NPCommPassword=1234567890ABCD

# Set the magic phrase for the group.

NPCommMagic=E-Case Works!

#### User ID

This is the ID assigned to a particular remote user.  For better security, these should be assigned individually to allow unique passwords and Ids for each user.  Assigning unique ID's allows a given user's ID to be revoked or suspended without affecting the entire remote sales force.  It will take longer to create the accounts this way, but it will simplify removing (denying access) a user who should no longer have access to the system.  Using unique user ID/password combinations is recommended.

Everyone in a given sales group could be assigned the same user ID.  This reduces the security level but makes the initial creation of the accounts easy.  The trade is that if and when a user needs to be removed or denied access to the system, the remaining sales force would all have to have their ID's and passwords changed.

The user ID is set on the remote pen unit in the environ.ini file with the environment variable NPCommUserId=ABC.

#### Domain (Group)

This is really just the name of the server.  This server name MUST be set correctly on the remote pen unit environ.ini file with the environment variable NPCommGroupId=MobilePoint.

#### Password

This is self-explanatory.  The password is required to successfully login.  This is set on the MobilePoint remote pen unit using the environ.ini setting NPCommPassword=1234567890ABCD.  Note that in order to successfully connect, the pen unit user ID and password must be valid on the Domain (group) NT server.

#### Inbox

This has been implemented through the upload market intelligence command.  The upload directory is specified in the file section.cfg.  The actual directory name is not important as long as the service has the service has the authority to read and write to that directory.  The "inbox" is thus a global inbox and is NOT specific to a particular user.  Each of the uploaded files is named using the users ID as the first characters of the filename.

User specific inboxes can be implemented and created using the HQ Comm DLL on the MobilePoint remote pen unit and executing the "Put File" command after login.

#### Outbox

This has not yet been implemented in the initial version of the server.  Support for the function is in place.  User specific outboxes can be implemented and created using the HQ Comm DLL command on the MobilePoint remote pen unit and executing the "Get File" after login.

#### Section Upload

In order to have the upload section menu entry enabled in the HQ Link application on the MobilePoint remote pen unit, the environment variable HQAdminPriv=yes must be set.  The default value is HQAdminPriv=no.

# System Operations

## Stopping Gmodem Service

Start the system service manager.  Select the desired gmodem service to stop.  The name will be of the form "Gmodem Service Com[N]", where N is a number from 1 to maximum number of COM ports installed.  After the service has been selected, hit the "Stop" button and wait for the service to be stopped.

## Starting Gmodem Service

Start the system service manager.  Select the desired gmodem service to start.  The name will be of the form "Gmodem Service Com[N]", where N is a number from 1 to maximum number of COM ports installed.  After the service has been selected, hit the "Start" button and wait for the service to be started.

## Removing Service(s)

The gmodem service for a particular COM port can be removed very easily.

Create a DOS Prompt Box session.

Change directory to the service installation directory.

Type gmodemService -remove Com[N], where N designates the port number of the service to remove.  The service will be stopped and then removed from the service manager.

## Uninstalling Gmodem Service

First, remove every instance of gmodem service for every COM port.

Next, delete the installation directory for gmodem service.  Ususally this will be \Program Files\gmodemService.

Finally, use the registry editor to delete the key HKEY_LOCAL_MACHINE\SOFTWARE\MobilePoint\GmodemService.

At this point the gmodem service has been completely uninstalled.  The data and queue directories still exist and may also be deleted if desired.  Normally these queues and data files are in \queue.

## Testing Communications

Communications can most easily be tested using a remote pen unit with HQ Link installed.

#### Upload

First make sure that the user account and password and the group (domain) names set up on the NT machine matches the settings in ALL 3 environ.ini files on the pen unit.  Set the HQAdminPriv=yes. It is easiest to modify 1 and overwrite the other 2.  The environ.ini files are in \penpoint\boot\environ.ini, \mp\fuji\environ.ini and \mp\penpoint\boot\environ.ini.

Start the gmodem service on the NT server.

Make sure the "auto-answer light" turns on.

Make sure an analog phone line is connected to the modem.

Boot E-case on the pen computer.

Create an HQ Link section.

Open HQ link.

Choose Options.

Choose dialing location and set up appropriately for the location.

Choose "Comm Configuration".  Set the phone number for the NT server.

Make sure "Modem" is checked and that "Serial" is not checked.

Copy the "Read Me" document into the HQ Link section.  Rename it to "Read Me 1".

Put a check mark in the load box for the "Read Me 1" document.

From the "Update" menu choose  "Upload Checked Documents".

The communication process should start.  You should briefly hear the dial tones and the modem lock up signals.  The "carrier" light should lock up on the NT Server modem.

Once carrier lock is established the pen unit should log in.  Provided the magic phrases, user ID's and passwords are set properly, the login should complete successfully.  After login, the compressed tar file of the "Read Me 1" document should be transmitted to the server.

#### Download

Create a new HQ Link section on the pen unit.  Rename it to "Download".

Turn to the "Download" section.

The phone numbers and dialing location should be the same as previously used.

Select "Full Update" from the "Update" menu.

The pen unit should dial the server and successfully login, download and import the "Read Me 1" document.

Turn to the "Read Me 1" document.  It should match the original document uploaded to the server.

#### Market Intelligence

Prior to this test, a test file named \mp\gqueue\upload.log must be created.  For test purposes, this can be any file or file type.  The only requirement is that it is named properly.  In the event that the HQ Link application has been set up without the misc\catalog.ini file, the test file should be named  \upload.log, and will need to be in the root directory.  Creating a file named misc\catalog.ini with a single entry as follows:

UPLOAD_LOGFILE=gqueue\upload.log

can be used to control where HQ Link attempts to pull the file upload.log for transmission.   In the example above, the upload.log file would be assumed to be in \boot\gqueue\upload.log.  Upon translation by the MobilePoint file system, the would equate to \mp\gqueue\upload.log since the penpoint root directory is \mp.

After verifying that the upload.log file exists, re-open either "HQ Link" or "Download".

Select the "Market Intelligence" menu entry from the "Update" menu.

The file upload.log will be transmitted to the Server.  An "OK" button will appear with a message stating  that the transmission was either a success  or a failure.  If the login has been setup properly, this will succeed.

Click OK.  Sometimes a box outline is left on the screen.  Simply turn back to the table of contents to clear it.

On the server side directory designated for "Market Intelligence" will contain a file named <USERID>_<TIMESTAMP>.upl.  The default directory for receiving uploaded log files is \queue\db\mat.

## Section List Format

The section list is stored in the directory RECV_SEC_DIR=\queue\db\sec by default.  The filename is section.ini.  This file is stored as an array of the following structure:

typedef struct SEC_ENTRY

{

U32             classId;

U32             kbytes;

//

//  These are additional structure information (32 bytes) to be used when

//  adding timestamp, author, group and "modified" information for the

//  section entries.  To be incorporated later.

U32     secCRC;

U32             createTime;

U32             modifiedTime;

U32     reserved0;

U32     reserved1;

U8              authorID[12];

U8      groupID[12];

//

U8              ppname[32];

U8              loc[256];// Space for very long path name, relative to directory of ini file.

} SEC_ENTRY, * P_SEC_ENTRY;

An administration tool (to Delete or Add entries) for the section list on the NT server could easily be constructed using the above structure definition and Visual Basic application development tools.