Technical Support Note #1					mp’96 1.0

Hardware Checkout

2/27/96

These procedures should be perform before sending new tablet hardware out to the field.

TIII Harddisks - check out in this order (the order is important!)

1.	visual inspection, any dents are cause for rejection.

2.	format using DOS Format

3.	run Norton disk doctor check (include full surface check)

Modem Checkout

1.	Place modem in slot 2 of the 730T or TE

2. Connect a data line to the modem

3.	From the maintenance menu run "Test Modem”

4.	You should see "Card Resetting...",then some output, then you will pause.  Any messages regarding Fail, Error or Abort should be recorded.

5.	Hit any key, after some diagnostic output (including a series of four ‘OK’s) and20-30 seconds you should see the message "Passed", hit any key to continue.

IBM730T/TE Checkout

1.	Turn the power completely off on the tablet.

2.	Insert memory card if not already inserted.

3.	Press and hold the Suspend button with the Pen Tip and then press and hold the White Power on switch, release when you see the "Copyright" message on the screen.

4.	You should see the Setup menu (if not try start at step 1 again), Tap on the "Test" icon to go to the Test menu.

5.	Tap on the "All" icon in the upper left corner (test will run for about 10 minutes)

6.	You should see a circle over all the tests performed meaning they passed.  If you see an X then that test failed.

*Note, if when you boot you see a large POST circle-cross displayed, record the error number below it and hit return.  You should go to the test menu mentioned in step 4 above.  Run test all and report findings to IBM Technical Support.