Leo One

Public Application Programming Interface

Functional Requirements Specification

DRAFT

Specification No.  xxxxxx

REVISION 0.0

Monday, January 29, 2001

Approvals

Program Technical Director					Date

Network Segment						Date

Revision History

| Revision Level | Date/Name | Change |
| --- | --- | --- |
| Created | 1/25/01 CEW |  |

# Purpose

This Functional Specification is a marketing driven document which outlines the major requirements and objectives towards providing a Leo One Public Application Programming Interface (PAPI) and development framework.   The goal of the PAPI and framework is to facilitate application development and provide the foundation for numerous industry specific solutions in the Leo One target markets.

# Scope

This document is intended for those concerned with application development using the Leo One network.  It is not intended to provide a complete understanding of the Leo One Network architecture and its lower level interfaces.  It also does not specify industry-specific or higher-level development kits, though does refer to these as example.  The reader is expected to be familiar with the concepts of a Short Message Service (SMS) and Short Messaging Service Center (SMSC) which the PAPI framework is modeled upon.

The following documents are referenced:

- [Leo One Network Segment Spec] 	description of Leo One Network architecture.

- [Leo One protocol Specification] 	description of Leo One Network protocol.

- [GSM SMS Point to Point 03.40]	description of the point to point Short Message Service

- [SMPP Protocol Specification v3.4]	description of the Short Message Peer to Peer Protocol

- [SMPPP Protocol Specification v1.1]	description of the SMPP Provisioning Interface Spec.

# Concept Overview

## The "Leo One Global Short Message Service"

Leo One is a small, low-earth orbiting satellite constellation designed for reliable two-way short message delivery.  The 48-satellite Leo One constellation provides 24-hour coverage and the capability for near real-time operation on a global basis.  It will be optimized to address the market for Short Messaging Service (SMS) solutions that place high value on a single worldwide network dedicated to business requirements.

SMS is proving to be an ideal lightweight communication solution for many applications and application protocol standards.  Leo One will support SMS standards and solutions with a singular set of value added services characterized by:

- Superior availability (global and ubiquitous coverage)

- Superior reliability (e.g. all messages are acknowledged)

- Enhanced SMS functionality (e.g. Multicast)

- Secure communications (multiple layers of encryption)

## How It Will Work

Though a sophisticated satellite technology, Leo One is a straightforward Global SMS architecture, much less complex than current terrestrial methods that support roaming via a complex interweaving of competing networks and switching stations originally design for voice services.  As shown in Figure 1, subscriber devices anywhere in the world will be in constant radio view of the orbiting satellites.  Satellites will communicate via a ground-based network infrastructure and set of receiving antenna stations, approximately 20, placed worldwide.

Figure 1: The Leo One Global SMS Architecture

Short messages to and from subscriber units are managed by Global Short Messaging Service Centers (SMSC), licensed and operated by authorized Leo One service providers.

### Leo One Global Short Message Service Center

Like all SMS architectures, Leo One Global SMS is a store and forward messaging service, where all messages and message services are centrally managed by a Short Message Sevice Center (SMSC).  Below in Figure 2 depicts a physical architecture of a basic Leo One service provider facility. The Leo One Home Gateway like any other network specific gateway can be integrated into a standard SMSC product.

Figure 2 - The Leo One Service Provider Facility

As shown in Figure 3 , depicting a typical SMSC product architecture, many SMSC vender offerings work with different network types (such as GSM and CDPD) and allow easy integration and management of multiple network gateways.

The typical off-the-shelf  SMSC will collect billing information, monitors message traffic and controls access for each of its supported networks.

Figure 3 - Logica's Telepath SMSC Supporting Multiple Networks

# Solution Concept

## Objective

The main objective is to provide developers and solutions providers an easy development path using existing short message services, standards and tools.  Thus, the requirements of a  Public Application Programming Interface (PAPI) is by definition driven by the services and facilities of an SMSC, and conforms to standard SMS interfaces.  From the developer's perspective,  the programming interface to Leo One is the same as the interface to a typical SMSC supporting any other SMS network.   A Leo One enabled SMSC gives services provider the ability to offer a true Global service that is cost effective and reliable.

As shown in Figure 3 , the underlying complexities of the Leo One network, satellites, protocols and gateway architecture are abstracted from the application on both the host and device end via standard SMS interfaces.

Figure 3 - Applications use SMS interfaces to access Leo One services

## Customer Scenario

### FleetBoard

Customers of Leo One service providers (or Leo One service providers themselves) will typically be "solution providers" for specific corporate application needs.

An example of a solution provider today (often also called an Application Service Provider or ASP) that will benefit from Leo One service Mercedes Benz's FleetBoard, a solution provider for the trucking industry.

Today FleetBoard provides Web host application services for corporate customers to manage their fleet operations. They are a full solution provider, providing FleetBoard hosted application, management and connectivity services to and from FleetBoard equipped vehicles.

The Web hosted application services provides all the features and power that a custom client server solution would provide within a corporation, but without the cost and complexities of managing it internally.   Corporate users at the enterprise integrate their backend databases via standard Internet technologies, and run applications hosted via any Web enabled platform (e.g. a PC), such as the service application shown here running within a Netscape browser.

Figure 4 - A FleetBoard Web Client Application

The FleetBoard vehicle application provides many onboard capabilities as shown here.

Figure 5 - The FleetBoard Vehicle Application

An embedded computer in the cab manages the various sensors, devices and the Human Machine Interface  (HMI), and communicates to and from FleetBoard's host service via an SMS communicator (i.e. GSM modem).

A architecture of the vehicle application shown in Figure 6 shows the serial line connection to a GSM communicator, is controlled via a standard AT modem device control language.

Figure 6 - FleetBoard Vehicle Application Block Diagram

How do SMS messages get transferred to and from the host and customer enterprise? The diagram below shows the big picture from device to customer host end.

Figure 6 - FleetBoard Vehicle Application Block Diagram

As shown, the application on the vehicle interfaces with the GSM communicator via an AT command library. SMS messages are passed through the GSM mobile telephone network or networks eventually received and stored by a destination SMSC.

The SMSC the forwards the message on to the solution provider's (in this case FleetBoard's host server) via a standard TCP/IP connection link.

The interface between the SMSC to the FleetBoard server could be one of several specific standard Host-to-SMSC interfaces supported by various vendors.  FleetBoard uses third party middleware messaging software (called ACL from Akumiitti Ltd.) that supports the various SMSC interfaces.   However, the non-proprietary Short Message Peer to Peer  (SMPP) interface has become the defacto standard and is supported by virtually all the SMSC product offerings, therefore most developers will not require the middleware.

### Using the Leo One Service

Guaranteeing the quality of service that Fleetboard's customer's require presents many challenges.   The are many issues with providing a high quality global service built upon a voice-oriented infrastructure made up of many local networks with competing interests. Though, SMS roaming does exist and works well enough for many consumers' needs just as cellular voice services do, sending and receiving of SMS across multiple networks is fraught with complexity and many points of failure.

No coverage, and roaming outside of the subscibers home network, not only has some technical issues, but it also increases costs (roaming charges) and perhaps most encumbering is it puts the solution provider into the business of dealing with multiple networks and assuring service with cooperative agreements. Huge issues since there can be dozens of networks to deal with as shown in the following diagram of Europe.

Thus, there is a need for global SMS by the solution provider to offer predictable reliability and cost on a worldwide basis.

Physically adding Leo One to the vehicle solution simply means adding additional another serial port connection to the Leo One communicator.  To be as compatible with the application software, the control and command language of the Leo One communicator would look like a GSM modem as much as possible.

Figure 6 - FleetBoard Vehicle Application Block Diagram

The FleetBoard application now has two service options (Leo One and GSM) as shown below.  Depending on the solution requirements and business criteria, the application will determine which service to use before sending messages.

Note there are two separate network services and paths shown here, no intermingling of messages between networks within the system,  no magical routing decisions passing SMS messages between each different network type.  Applications either use one or the other network service.

Other User Cases

Some solution providers will not require or want terrestrial SMS networking.  For example Daimler/Chrysler considers its core emergency telematics solution to be very cost sensitive.  The added cost of embedded terrestrial communicator is not practical. Leo One as the only practical global In this case the architecture is practically the same as shown below.

The AT SMS interface provides many functional queries to assist in the selection, for example signal strength and whether the home network is available (always in the case of Leo One).

Serial AT Cellular/SMS interface

Global SMSC -

Support for TCP/IP connectivity

Support for SMPP interface to and from host

The Short Message Peer to Peer (SMPP) protocol is an open, industry standard protocol

designed to provide a flexible data communications interface for transfer of short message data

between a Message Center,	3.4 SMPP

1.1 of SMPPP

Non-Requirements

A GSM/SMS phone does not send a message to a Leo One terminal nor vice versa.  I.e. no routing or switching decisions are required between different types of SMS networks. Applications themselves on both ends decide which network service to use using the functional interfaces defined.

The SMPP allows for native addressing. We will use the Leo One addressing format.

## Common Message Service Requirements

### Message Generation requirements

#### Scope

##### Leo One domain

#### Message persistence

#### ID requirements

##### Uniqueness within system

###### VHG definition

###### Sequence number

#### Class of Service

##### Escalation of priority

##### Definition of service epoch time

##### Queue relationship

##### Status/Query

##### Common reference

#### MDR Interface

##### MDR Constituents

##### Procedure

### Message Termination requirements

#### Class of Service

##### Definition of termination within system

##### Lack of compliance

###### Troubleshooting?

###### Exception reporting

#### MDR interface

##### Procedure

### Message Monitoring/Tracking requirements

#### Terminal end

#### Enterprise end

#### NMC/HG internal to stack

### Message Characteristics

#### Message size

#### Terminal Introduced

#### HG introduced

#### Message reliability

#### Message Integrity

#### Location of transmitter?

### Message Handling Requirements

#### Handshaking/Acknowledgments

##### TSL

###### TSU

###### TSD

##### GSL

###### GSU

###### GSD

##### Backbone

#### Network Element Routing of Messages

#### Admission Control

##### NMC Role

##### HG

##### Terminal

##### Satellite

#### Data Retention at network elements

##### Memory Requirements

##### Redundancy

#### Network Element Throughput Requirements

##### Terminal

##### Satellite

##### SG

##### HG

### Security

#### Link Encryption

##### TSD

##### TSU

##### GSD

##### GSU

##### Backbone

#### End-to-End Encryption

##### HG to Terminal

#### Message Sender Authentication

#### Data Privacy

##### Storage while at network element

##### VPN

#### CALEA-derivations

##### HG Access

##### Procedures

###### Initiation

###### Data handling

###### Termination

### PAPI

#### Capabilities

#### User API/System Tool Kit Development Support

### Enterprise Support

### Use of PAPI

#### Service type registration

##### Terminal End

##### HG End

## Enterprise to Terminal Messaging

### Unicast

#### Description

#### Near real-time (< 5 minute) delivery

##### fixed and mobile common requirements

###### acknowledgements, return receipts

##### fixed only requirements

##### mobile only requirements

###### location accuracy

#### Non real-time (5 to 30, > 30 minute) service

##### fixed and mobile common requirements

###### acknowledgements, return receipts

##### fixed only requirements

##### mobile only requirements

###### location accuracy

### Multicast

#### Description

#### Near real-time (< 5 minute) delivery

##### fixed and mobile common requirements

##### fixed only requirements

##### mobile only requirements

###### location accuracy

#### Non real-time (5 to 30, > 30 minute) service

##### fixed and mobile common requirements

##### fixed only requirements

##### mobile only requirements

###### location accuracy

### Polling

#### Description

#### Enterprise/User Requirements

#### NMC Requirements

#### Satellite Requirements

#### Terminal/Application Requirements

#### Near real-time operation

#### Non real-time operation

## Terminal to Enterprise Messaging

### Near real-time (< 5 minute) delivery

#### fixed and mobile common requirements

##### access method(s)

##### acknowledgements, return receipts

#### fixed only requirements

#### mobile only requirements

##### location accuracy

### Non real-time (5 to 30 and > 30 minute) service

#### fixed and mobile common requirements

##### access method(s)

##### acknowledgements, return receipts

#### fixed only requirements

#### mobile only requirements

##### location accuracy

## Two-Way Messaging

### Declaration of Architecture

### Services provided

### Applications-role

## System Wide Considerations

See GSM 7.05 Equipment (DTE - DCE) interface for Short Message Service (SMS)